Programming HP Robots Otto with MicroPython

Developer Guide for Thonny IDE with ESP32‑WROOM

HP Robots Otto was originally designed to run MicroPython, leveraging the ESP32 REPL for fast iteration and a beginner‑friendly workflow.

Using Thonny IDE, you can write, run, and test code instantly on the robot without reflashing, making development much smoother and educational.

MicroPython is recommended for new users, classrooms, and rapid prototyping. While Arduino C++ provides deeper control, it requires full board erasing and flashing each time you switch languages.

You can move between MicroPython and Arduino, but switching back to MicroPython requires reflashing MicroPython firmware onto the ESP32‑WROOM.

# 1. Requirements

Thonny IDE (with MicroPython support)

MicroPython firmware for ESP32‑WROOM

USB cable for connection

Install ‘esptool’ if flashing manually

# 2. Setting Up MicroPython on Otto

Download ESP32‑MicroPython firmware 1.6.1 (.bin)

Connect Otto to your computer via USB

Thonny → Tools → Options → Interpreter → MicroPython (ESP32)

Click ‘Install MicroPython’ to flash automatically

# 3. Basic MicroPython Examples

## 3.1 Built‑in LED (GPIO 2)

from machine import Pin
import time
led = Pin(2, Pin.OUT)
while True:
    led.value(1)
    time.sleep(0.5)
    led.value(0)
    time.sleep(0.5)

## 3.2 NeoPixel Ring (GPIO 4)

from machine import Pin
import neopixel, time
np = neopixel.NeoPixel(Pin(4), 13)
while True:
    for i in range(13):
        np[i] = (255,0,0)
        np.write()
        time.sleep(0.05)
    for i in range(13):
        np[i] = (0,0,0)
    np.write()

## 3.3 OLED Display (I2C GPIO 21/22)

from machine import Pin, I2C
import ssd1306
i2c = I2C(0, scl=Pin(22), sda=Pin(21))
d = ssd1306.SSD1306_I2C(128, 64, i2c)
d.text('Hello Otto!', 0, 0)
d.show()

## 3.4 Digital Encoder (GPIO 16)

from machine import Pin
import time
enc = Pin(16, Pin.IN, Pin.PULL_UP)
count = 0

def handler(pin):
    global count
    count += 1

enc.irq(trigger=Pin.IRQ_FALLING, handler=handler)

while True:
    print(count)
    time.sleep(0.5)

# 4. Running Code in Thonny

- Open Thonny, ensure MicroPython (ESP32) interpreter is selected

- Write code in the editor and click Run → executes instantly over REPL

- Save to device as main.py to auto‑run at boot

# 6. HP Robots Otto – MicroPython Libraries

A complete collection of MicroPython libraries, example programs, 3D STL files, and resources for use with the https://hprobots.com/otto-robot/product/.
Compatible with MicroPython v1.26.1.

👉 Recommended: Use our Quick Flasher to install MicroPython easily:
https://hprobots.com/otto-code/webcode/pythonuploader/pythonuploader.html

If you prefer using Arduino IDE, follow the Arduino alternative programming guide included in this repository.

📌 Repository Structure

This repository includes:

- MicroPython libraries for each robot component

- Example code for both block-based and text programming

- Wiring diagrams and pinouts

- 3D STL files for expansions

- Educational resources for transitioning from blocks to text coding

📁 wirings/

Circuit diagrams and “default wiring” reference used across all examples.

https://github.com/cparrapa/micropythonlibs/blob/main/wirings/Otto%20circuitboardpinout.png

Main Board GPIO Pinout

Always verify that the pins used in your code match the connector you are using:

| Function | GPIO | Notes |
| --- | --- | --- |
| Built-in LED | 2 | Status LED |
| Built-in Buzzer | 25 | Piezo buzzer |
| Battery voltage (ADC) | 39 | Battery sense |
| Connector #1 | 18, 19 | Ultrasonic / SPI OLED |
| Connector #2 | 16, 17 | MP3 module / Digital encoder |
| Connector #3 (I²C) | 22 (SCL), 21 (SDA) | Matrix, accelerometer, OLED (I²C) |
| Connector #4 | 26 | Tilt switch / Button |
| Connector #5 | 4 | RGB LED ring (13 LEDs) / Temp sensor |
| Connector #6 (ADC) | 32 | Line sensor L / Microphone |
| Connector #7 (ADC) | 33 | Line sensor R / Potentiometer / LDR |
| Connector #8 | 27 | Servo angle right |
| Connector #9 | 15 | Servo angle left |
| Connector #10 | 14 | Wheel servo (left) |
| Connector #11 | 13 | Wheel servo (right) |

📁 libraries/

MicroPython modules organized by functionality.
⚠️ Important: Examples require that you upload all library files to the Otto board first using Thonny or any MicroPython IDE.

Core Modules

| File | Description |
| --- | --- |
| ottomotor.py (v2.0) | Servo motor classes (split pending) |
| ottoangle.py | Angle-servo utilities |
| ottowalkroll.py | Biped + wheeled movement behaviors |
| ottobattery.py | Battery monitoring |
| otto4wd.py | 4-wheel drive (not available as blocks) |
| ottobuzzer.py (v2.2) | RTTTL melodies + piezo tones |
| ottoline.py | IR line sensors |
| ottoneopixel.py (v2.3) | RGB LEDs and NeoPixel devices |
| ottoring.py | ❗ Pending: 13-LED ring wrapper |
| ottomatrix.py | ❗ Pending: 8×8 LED matrix wrapper |
| ottoultrasonic.py | ❗ Pending: ultrasonic + LED combo |
| ottosensors.py (v2.1) | Multi-sensor utilities (split pending) |
| ottodht.py | DHT11 temperature/humidity |
| ottoldr.py | ❗ Pending: Light sensor |
| ottomic.py | ❗ Pending: analog sound detection |
| ottotilt.py | ❗ Pending: tilt switch utilities |
| ottobutton.py | ❗ Pending: button debouncing |
| ottoencoder.py | ❗ Pending: encoder abstraction |
| adxl345.py (v1.0) | Accelerometer driver |
| ottoaccelerometer.py | ❗ Pending: high-level accelerometer class |
| ssd1306.py (v0.0) | OLED driver (third-party) |
| ottodisplay.py (v2.1) | Faces/emoji rendering for OLED |
| ottomp3.py (v1.0) | MP3 module control |
| ottoble.py (v2.1) | Deprecated BLE communication |
| ottoiot.py (v1.0) | Wi-Fi IoT features |
| directory.py | Index file |

📘 Naming Conventions

To maintain consistency:

- Classes → CamelCase

- Functions → CamelCase

- Variables → snake_case

- Files/Modules → snake_case.py

- Constants → UPPER_CASE_WITH_UNDERSCORES

📁 blocks examples/

Examples compatible with the WebCode block editor (icons or words):
https://hprobots.com/otto-code/webcode/webcode.html

📘 Best beginner guide (highly recommended):
https://hprobots.com/otto-robot/code/#flipbook-hp-robots-otto-starter-coding-intro/1/

🧩 Goal:
Every example should have both a block version and a MicroPython text version to help students transition smoothly.

📁 code examples/

Full MicroPython demo programs organized by component/sensor module.

| Folder | Status | Notes |
| --- | --- | --- |
| accelerometer | TESTED | ADXL345 examples — needs more games |
| biped | WIP | Classic Otto walking |
| bluetooth | TESTED | BLE WebControl & WebCode |
| botbit | WIP | Requires adaptation to HP Otto pins |
| button | TESTED | Simple digital input |
| buzzer | TESTED | Piezo melodies |
| color | WIP | TCS34725 sensor (EOL chip) |
| encoder | WIP | Needs ottoencoder.py integration |
| IoT | WIP | Wi-Fi examples |
| koi | WIP | AI camera integration |
| light | TESTED | LDR analog sensor |
| line | TESTED | Line tracking IR sensors |
| matrix | TESTED | LED matrix examples (missing mouths/emojis) |
| microphone | TESTED | Basic analog sound detection |
| motorangle | TESTED | 180° servo examples |
| motorwheel | TESTED | 360° wheel servos |
| mp3 | TESTED | MP3 module (needs separate lib) |
| ninja | TESTED | Walk-and-Roll hybrid |
| oled | TESTED | 128×64 display — more faces needed |
| omni | WIP | From Alex remix |
| potentiometer | TESTED | Analog input |
| ring | TESTED | RGB LED ring + built-in LED |
| temphumi | TESTED | DHT11/22 |
| tilt | TESTED | Digital tilt switch |
| showcase | TESTED | Demo bundle |
| ultrasonic | TESTED | Distance sensor (needs more examples) |
| wifi | WIP | Web server + Wi-Fi control |

💡 New components, example contributions, and expansions are very welcome!